Q19. Difference between analogWrite() and analogRead(). What is PWM? Give IoT examples. (2 Marks)

Difference between analogWrite() and analogRead()

| analogWrite() | analogRead() |
| --- | --- |
| Used to generate PWM output. | Used to read analog voltage from a sensor. |
| Writes a value from 0–255. | Reads a value from 0–1023. |
| Works on PWM pins (3, 5, 6, 9, 10, 11 in Arduino Uno). | Works on Analog pins (A0–A5). |
| Controls brightness of LEDs or motor speed. | Reads data from sensors like LDR, temperature sensor, and potentiometer. |

What is PWM?

PWM (Pulse Width Modulation) is a technique used to simulate analog output by rapidly switching a digital pin ON and OFF. By changing the duty cycle, the average output voltage changes.

Practical IoT Examples

- analogWrite() Example: Adjusting the brightness of a smart street light LED based on ambient light.

- analogRead() Example: Reading values from an LDR sensor in a smart home lighting system.

- PWM Example: Controlling the speed of a DC fan in a smart temperature control system.

Q20. Explain setup() and loop() functions. What happens if a long delay() is used? What is the non-blocking alternative? (2 Marks)

setup()

- The setup() function runs only once when the Arduino starts or resets.

- It is used to initialize pins, sensors, and serial communication.

loop()

- The loop() function runs continuously after setup() finishes.

- It contains the main program logic that executes repeatedly.

Effect of a Long delay() in loop()

A long delay() stops the Arduino from executing other tasks during that time. As a result:

- Sensor readings become slow.

- Button presses may be missed.

- The system becomes less responsive.

Non-blocking Alternative

The recommended non-blocking alternative is to use the millis() function.

millis() allows multiple tasks to run simultaneously without stopping the program, making IoT applications more responsive and efficient.

Practical Example

A smart irrigation system can use millis() to:

- Read the soil moisture sensor every second.

- Update the LCD display.

- Check Wi-Fi connectivity.

- Control the water pump